How to Capture High Resolution Image Using DCM130

- Using DCM130 with software PhotoShop

  - Double click the icon “PhotoShop” to operate it.

  - Click “file > Import > Mega PCCAM (MC1300)”

  - Then following video screen appears on your computer screen.

Exit

For making a video

For capturing a still image 1280X1008

Setting

  - Click “Setting”, you can select the resolution from 160X120 to 640X480. Once different size is selected, the video screen will change accordingly to the size selected. But this is only for selecting the video image size, not workable for the still image size.

  - Pressing “Capture”, you can capture a still image from DCM130. No matter what size you selected, the still image captured from DCM130 is 1280X1008.

- Using DCM130 with software ACDSee

  - Double click the icon “ACDSee” to operate it.

  - Click “file > Acquire Setup > Mage PCCAM (MC1300)”

  - Click “file > Acquire”

  - Then a video screen appears on your computer screen. It is the same as above in PhotoShop.

- Using DCM130 with software MiniSee

  - Click the button “Acquire” and a video screen same as above appears. Pressing the button “Capture”, you can capture a still image of size 1280X1008.

  - Click the button “Video” and a video screen appears. After selecting the size between 160X120 and 640X480, by pressing the button “Capture”, you can capture a still image of the size from 160X120 to 640X480.