SCOPETEK®     www.scopetek.com     Hangzhou ScopeTek Opto-Electric Co.,Ltd.

Digital Cameras with C-mount

DCMC Series

DCMC is a digital camera with C-mount. Each camera comes with a mini disc including the software MiniSee and ScopePhoto. By using the software MiniSee, it is very convenient to preview live image and to capture pictures. The advanced software ScopePhoto included with the camera ensures simple and convenient operation on the video for preview and images captured.

DCMC is USB2.0 version, so please make sure that it is plugged into USB2.0 port in PC.

Note:

- All detailed function should refer to the software MiniSee, SCOPETEK has the rights to change the function without notifications.

- All detailed function should refer to the software ScopePhoto, SCOPETEK has the rights to change the function without notifications.

- Please keep contact with our distributors for the further information about other models as follows:

| Cameras for microscope | Cameras with C-mount | Cameras for telescope |
| --- | --- | --- |
| ACM35 (PAL, 220V) ACM35 (NTSC, 110V) DCM35-A, DCM35-B DCM130-A, DCM130-B DCM200-A, DCM200-B DCM300-A, DCM200-B DCM500-A, DCM500-B | DCMC35 DCMC130 DCMC200 DCMC300 DCMC500 | ACT35 (PAL, 220V) ACT35 (NTSC, 110V) DCT35-A, DCT35-B DCT130-A, DCT130-B DCT200-A, DCT200-B DCT300-A, DCT300-B DCT500-A, DCT500-B |

SCOPETEK®   www.scopetek.com    Hangzhou ScopeTek Opto-Electric Co.,Ltd.

Digital Cameras with C-mount:

DCMC Series

| Model | DCMC35 | DCMC130 | DCMC200 | DCMC300 | DCMC500 |
| --- | --- | --- | --- | --- | --- |
| Image Sensor | 1/3” CMOS | 1/2” CMOS | 1/3” CMOS | 1/2” CMOS | 1/2.5” CMOS |
| Pixel Size | 8μm X 8μm | 5.2μm X 5.2μm | 2.8μm X 2.8μm | 3.2μm X 3.2μm | 2.2μm X 2.2μm |
| Sensitivity | 2.0v/lux-sec @550nm | 1.8v/lux-sec @550nm | 1.0v/lux-sec @550nm | 1.0v/lux-sec @550nm | 1.0v/lux-sec @550nm |
| Max Resolution (Hardware) | 640 X 480 0.3M pixels | 1280 X 1024 1.3M pixels | 1600 X 1200 2M pixels | 2048 X 1536 3M pixels | 2592 X 1944 5M pixels |
| Speed (Depend on the PC) | 30frames/sec @ 640 X 480 | 20frames/sec @1280 X 1024 | 6frames/sec @1600 X 1200 | 4frames/sec @2048 X 1536 | 2frames/sec @2592 X 1944 |
| Dynamic Range | 62dB | 62dB | 71.5dB | 62dB | 75dB |
| Wave Length (nm) | 400-650nm (IR-Cut added) |  |  |  |  |
| Exposure | ERS (Electronic Rolling Snap) |  |  |  |  |
| White Balance | Automatic / Manual |  |  |  |  |
| Auto-exposure | Automatic / Manual |  |  |  |  |
| Image Output | USB2.0, 480Mb/s |  |  |  |  |
| Power Supply | USB2.0 |  |  |  |  |
| Programmable Control | Image size, brightness, gain, exposure time |  |  |  |  |
| Working Temperature | -30°C ~ 70°C |  |  |  |  |
| Lens mount | C / CS mount |  |  |  |  |
| Camera body | Metal body painted in black |  |  |  |  |
| Camera Size | 41 X 50 X 50 (mm) |  |  |  |  |
| USB Cable | USB cable, length 1.5m |  |  |  |  |
| Operation System | Windows 2000 and Windows XP (SP2) |  |  |  |  |
| Software | Image-processing software “MiniSee” and “ScopePhoto” Driver, which supports DShow & TWAIN interface, output data RGB24 |  |  |  |  |

SCOPETEK®     www.scopetek.com    Hangzhou ScopeTek Opto-Electric Co.,Ltd.

Digital Cameras with C-mount: DCMC Series

Introduction about software ScopePhoto

ScopePhoto is the image-processing software professionally designed for SCOPETEK digital cameras, including DCM, DCMC and DCT Series. Except the basic image file operation and processing, the ScopePhoto has the following characteristic specially designed for the scientific analysis of the captured image through the camera or other image capture devices. The main functions of the ScopePhoto are listed as follows (9I’s):

  - Image archival with measurement data in layer format (*.SFT format, other formats are also supported)

  - Image capture through Direct Show and Twain interface

  - Image edit operations.

  - Image view operations, include Image browsing in window explorer format, image history operations, measurement results management, ruler and grid on image , image zooming and panning operations

  - Image basic operations like other similar software are included in the Image menu. At here you can do the adjustments on the image at ease

  - Image processing filter for noise reduction and enhancement (Image enhancement filters, edge enhancement filters, morphology operation filters, User defined kernel filter, include convolution and morphological filters)

  - Image layer operation specially designed for the measurement operation (Layer technique is used to separate the measured results and the different results can be put on different layers)

  - Image plug-in operation, with this menu, user can design his owns image processing algorithm at ease.

  - Image options menu for user customized operation characteristic.